Ravi Mann

ECE 362

Pre-Lab # 3

Introduction:

Purpose and Objective of the Lab:

- Equip with a comprehensive understanding of programming in assembly language for HC(S)12.

- Focus on accurate implementation of arithmetic instructions for computations.

- Emphasize scaling numerical operations for precise results.

- Differentiate between signed and unsigned computations, understanding their impact on the Condition Code Register (CCR).

- Learn to create software for seamless operation on the I/O board.

Concepts Covered in the Lab:

- Accurate implementation of arithmetic instructions.

- Scaling numerical operations for precision.

- Understanding signed and unsigned computations and their effects on the CCR.

- Creating software for continuous operation on the I/O board.

Lab 3.1.1:

Objective/Purpose:

Create an assembly language program that calculates the value of y in the given straight-line equation y = mx + b.

Expected Results:

The program should utilize the specified slope (m = 0.68) and offset (b = 12) and assume that the input value x is an unsigned 8-bit number. The result, which will always fit within 8 bits, should be stored in accumulator A. The lab aims to validate the correctness of the program by testing it with various values of x.

Code:

XDEF	Entry

XREF	__SEG_END_SSTACK

Variables:		Section

VAL:			ds.b	1		; defining Val variable space

RESULT:		ds.b	1		; defining result variable space

Constants:		Section

Slope			EQU	#68		; Slope value (0.68 * 100, represented as an integer)

Offset		EQU	#12		; Offset value (b)

Code:			Section

Entry:

LDAA		VAL		;to be scaled

LDAB		Slope		;B = 68

MUL				;D = A X B

LDX 		#100		;X = 100

IDIV				;D/X = X…D, quotient in X

TFR		X,D		;Move quotient to D

ADDD		Offset	;D = D + Offset

STAD		RESULT	;Store D into RESULT

END

nop

Lab 3.1.2:

Objective/Purpose:

Create an assembly language program that calculates the value of y in the given parabolic equation y=mx^2+b.

Expected Results:

The code performs the calculation of y=mx^2+b using the specified slope and offset values. The result is stored in register D, ensuring it fits within 16 bits. The program will be tested with various values of x to verify its accuracy.

Lab 3.2.1:

Objective/Purpose:

Explore the behavior of the Condition Code Register (CCR) in response to different arithmetic operations.

Expected Results:

The program will be run with various combinations of num_1 and num_2. After each run, the contents of Accumulator A will be recorded, and the states of the specified CCR bits (Carry, Overflow, Negative, Zero) will be observed and noted as either set or cleared. The contents of Accumulator A after each program execution will be recorded. The states of the CCR bits (Carry, Overflow, Negative, Zero) will be observed and noted as set or cleared.

Lab 3.2.2:

Objective/Purpose:

Write a short program that loads accumulator A with the value 0x12, defines a variable VAR_1 as a byte initialized with 0x30, and then uses the CMPA instruction to compare the value in accumulator A to VAR_1.

Expected Results:

Contents of Accumulator A after program execution will be recorded. States of the CCR bits (Carry, Overflow, Negative, Zero) will be observed and noted as set or cleared.

Lab 3.2.3:

Objective/Purpose:

Enter, build, and run specific programs. The value in accumulator A when the NOP instruction is reached will be observed and recorded.

Expected Results:

Contents of Accumulator A after each program execution (when NOP instruction is reached) will be recorded.

Lab 3.3:

Objective/Purpose:

Reads switch inputs and uses a lookup table to map them to corresponding LED outputs. The left value represents the switch input, and the right value represents the value to be sent to the LEDs.

Expected Results:

The program is expected to perform the following tasks of continuously reading the switch inputs and use the lookup table to determine the corresponding LED value for each switch input. The program should loop indefinitely, continuously monitoring and updating the LEDs based on switch changes.

ore submitting

Note:

- Pay attention to grammatical and spelling errors

- Use your own words (don’t copy the slides)

- Single spaced

- Code should be commented (useful and meaningful comments)

- Fonts and sizes:

  - Use the “Times New Roman” font or any similar font

  - Use font size 14 bold for headings

  - Use font size 12 for subheadings

  - Use font size 12 for text

  - Use the “Courier New” font for the code and the size should be 10