Ravi Mann

ECE 362

Post-Lab # 6

Introduction:

Introduction:

We explore basic ideas related to memory management, subroutines, and stack operations in this project. Our main goal is to provide you a thorough grasp of how the stack is implemented, how register contents are preserved, and how communication with subroutines is made easier. We will also go into the nuances of paging and banked memory, which are essential parts of the HC(S)12 design. It is crucial to understand that, because of its poor mobility across many CPU architectures, assembly language is not used for the majority of programming activities. The intention is to foster a greater understanding of the crucial function that the stack performs throughout program execution.

Lab 6.1:

Objective/Purpose:

The purpose of this code is to demonstrate stack operations and subroutine execution in the context of the HC(S)12 microcontroller. Specifically, it initializes the stack pointer and performs a series of operations involving the A, B, X, and Y registers.

Explanation:

The program begins by initializing the stack pointer (`SP`) to the end of the stack segment. It then loads values into registers A, B, X, and Y. The subroutine `StackRoutine` is called, which involves pushing these registers onto the stack, loading them with zeros, and then popping values back from the stack into the registers.

Results:

The code effectively showcases the process of pushing and popping values onto and from the stack. The output of this code will demonstrate that the values initially stored in registers A, B, X, and Y are successfully preserved and restored using stack operations. Additionally, the subroutine returns with the result in the D register.

| Stack Address | Byte Data |
| --- | --- |
| $500 | -- |
| 4FC | C0 |
| 4FD | 11 |
| 4FC | 22 |
| 4FA | 33 |
| 4F8 | 44 |
| 4F9 | 44 |
| 4FA | 33 |
| 4FC | 22 |
| 4FE | C0 |

Code:

; Include derivative-specific definitions

INCLUDE 'derivative.inc'

; export symbols

XDEF Entry, _Startup, main

; we use export 'Entry' as symbol. This allows us to

; reference 'Entry' either in the linker .prm file

; or from C/C++ later on

XREF __SEG_END_SSTACK ; symbol defined by the linker for the end of the stack

; code section

MyCode: SECTION

main:

_Startup:

Entry:

LDS 	#__SEG_END_SSTACK ; initialize the stack pointer

LDAA 	#$11

LDAB	#$22

LDX 	#$3333

LDY 	#$4444

JSR 	StackRoutine

StackRoutine:

PSHA 		; Push A register onto the stack

PSHB 		; Push B register onto the stack

PSHX 		; Push X register onto the stack

PSHY 		; Push Y register onto the stack

LDAA 	#$0 	; Load A register with value 0

LDAB 	#$0 	; Load B register with value 0

LDX 	#$0 	; Load X register with value 0

LDY 	#$0 	; Load Y register with value 0

PULA 		; Pop value from stack into A register

PULB 		; Pop value from stack into B register

PULX 		; Pop value from stack into X register

PULY 		; Pop value from stack into Y register

RTS 		; Return from subroutine, result in D

Lab 5.1.2:

Objective/Purpose:

The objective of this code is to demonstrate input and output operations in the context of the HC(S)12 microcontroller. Specifically, it aims to set up Port S to control LEDs and continuously monitor the state of switch 1 on Port T. When switch 1 is in the low state, a particular LED will be illuminated.

Explanation:

The program begins by defining some constants for memory addresses associated with Port S, Port T, and specific masks for operations. The section titled "Variables" is reserved for any data variables, though it is currently empty.

In the "MyCode" section, the main program starts at the label "start". It sets all the LEDs for Port S by setting the corresponding bits in the Data Direction Register (DDR_S). It then enters a loop labeled "loop_0high", where it continuously checks the state of switch 1 on Port T. If switch 1 is low, the program proceeds to the next label "loop_0low".

In "loop_0low", the program checks if switch 1 is high. If it is, the program jumps back to "loop_0high" to continue monitoring the switch state. If switch 1 is low, it advances to "loop_0high2".

In "loop_0high2", the program again checks the state of switch 1. If it is low, it sets the 6th bit of Port S, which corresponds to illuminating a specific LED.

The "NOP" instruction serves as a placeholder and does not affect program flow.

Results:

This code sets up the microcontroller to interact with external components connected to Port S and Port T. It continually monitors the state of switch 1. When switch 1 is in the low state, a specific LED on Port S is illuminated. The behavior observed will depend on the physical connections to Port S and Port T, as well as the external components connected.

Code:

; Include derivative-specific definitions

INCLUDE 'derivative.inc'

; export symbols

XDEF Entry, _Startup, main

; we use export 'Entry' as symbol. This allows us to

; reference 'Entry' either in the linker .prm file

; or from C/C++ later on

XREF __SEG_END_SSTACK      ; symbol defined by the linker for the end of the stack

Constants: Section

Port_s  	equ $248

Ddr_s   	equ $24A

Port_t  	equ $240

Mask    	equ $FF

Mask1    	equ $01

MyCode: SECTION

main:

_Startup:

Entry:

start:

BSET    Ddr_s, Mask  			; set all the leds for port s

loop_0high:

BRCLR   Port_t, Mask1, loop_0high 	;branch if switch 1 is low

loop_0low:

BRSET   Port_t, Mask1, loop_0low  	;branch if switch 1 is high

loop_0high2:

BRCLR   Port_t, Mask1, loop_0high2 	;branch if switch 1 is low

BSET    Port_s, #%01000000

NOP

Lab 6.2:

Objective/Purpose:

The objective of this program is to create a lookup table-based system for controlling LEDs based on the state of switches. It aims to read the switch input continuously, use a lookup table to map each switch state to a specific LED output value, and display the corresponding LED pattern on the LEDs connected to the microcontroller.

Explanation:

This program continuously monitors the state of switches connected to Port T. It uses a lookup table to determine the corresponding LED pattern and displays it on Port S. The program runs in an infinite loop, providing real-time interaction between switches and LEDs.

Results:

The program will initialize Port S for output. It will then continuously monitor the state of the switches connected to Port T. Whenever the switch state changes, the program will use the lookup table to determine the corresponding LED pattern and display it on Port S. The program will run in an infinite loop, providing real-time interaction between switches and LEDs.

Code:

XDEF  SendMessageInfo

XREF  SendsChr

SendMessageInfo:

LEAX  3, SP     ; Array

LDY   5, SP     ; Counter

Loop:

LDAA  1, X+

PSHA

CALL  SendsChr

PULA

LEAS  -1, SP

DEY

CPY   #0

BNE   Loop

RTC

Lab 6.3:

Objective/Purpose:

The objective of this code is to demonstrate the utilization of an HC12 microcontroller in conjunction with an LCD display and a potentiometer. Specifically, it reads the analog value from the potentiometer, converts it to a three-digit decimal representation, and displays it on the LCD screen.

Explanation:

The program begins with string initialization, creating a message "The value of the pot is:      " in the `disp` array. It then calls `init_LCD` to initialize the LCD display. In the main loop labeled "loop", the program continuously performs the following steps. Calls `read_pot` to read the analog value from the potentiometer.

Loads the potentiometer value into register D. Divides the value by 100, obtaining the hundreds place digit, which is then converted to ASCII and stored in the variable `HUND`. Divides the value by 10, obtaining the tens place digit, which is converted to ASCII and stored in the variable `TEN`. Divides the value by 1, obtaining the ones place digit, which is converted to ASCII and stored in the variable `ONE`. Updates the corresponding positions in the string `disp` with the ASCII representations of the digits.

Calls `display_string` to display the updated string on the LCD.

Results:

The code successfully reads the analog value from the potentiometer and displays the corresponding three-digit decimal value on the LCD screen. The displayed value will change as the potentiometer is adjusted. This code demonstrates effective interaction between the microcontroller, potentiometer, and LCD display, showcasing a practical application of the HC12 microcontroller.

Code:

;**************************************************************

;* This stationery serves as the framework for a              *

;* user application. For a more comprehensive program that    *

;* demonstrates the more advanced functionality of this       *

;* processor, please see the demonstration applications       *

;* located in the examples subdirectory of the                *

;* Freescale CodeWarrior for the HC12 Program directory       *

;**************************************************************

; Include derivative-specific definitions

INCLUDE 'derivative.inc'

; export symbols

XDEF Entry, _Startup

; we use export 'Entry' as symbol. This allows us to

; reference 'Entry' either in the linker .prm file

; or from C/C++ later on

XREF __SEG_END_SSTACK      ; symbol defined by the linker for the end of the stack

; LCD References

XREF  init_LCD

; Potentiometer References

XREF  display_string

XREF  pot_value

XREF  read_pot

; variable/data section

my_variable: SECTION

disp:	ds.b 33

HUND: ds.b 1

TEN: ds.b 1

ONE: ds.b 1

; code section

MyCode:     SECTION

Entry:

_Startup:

lds       #__SEG_END_SSTACK

;*********************string initializations*********************

;intializing string "disp" to be:

;"The value of the pot is:      ",0

movb #'T',disp

movb #'h',disp+1

movb #'e',disp+2

movb #' ',disp+3

movb #'v',disp+4

movb #'a',disp+5

movb #'l',disp+6

movb #'u',disp+7

movb #'e',disp+8

movb #' ',disp+9

movb #'o',disp+10

movb #'f',disp+11

movb #' ',disp+12

movb #'t',disp+13

movb #'h',disp+14

movb #'e',disp+15

movb #' ',disp+16

movb #'p',disp+17

movb #'o',disp+18

movb #'t',disp+19

movb #' ',disp+20

movb #'i',disp+21

movb #'s',disp+22

movb #':',disp+23

movb #' ',disp+24

movb #' ',disp+25

movb #' ',disp+26

movb #' ',disp+27

movb #' ',disp+28

movb #' ',disp+29

movb #' ',disp+30

movb #' ',disp+31

movb #0,disp+32    ;string terminator, acts like '\0'

;**************************************************************

jsr  init_LCD      ;a warning in the C code about posible loss of data is ok

;**************************************************************

;                   Write Your Code Here

loop:

JSR    read_pot

LDD    pot_value

LDX    #100

IDIV

XGDX

ADDB   #$30

STAB   HUND

TFR    X,D

LDX    #10

IDIV

XGDX

ADDB   #$30

STAB   TEN

TFR    X,D

LDX    #1

IDIV

XGDX

ADDB   #$30

STAB   ONE

TFR    X,D

MOVB   HUND, disp+25

MOVB   TEN, disp+26

MOVB   ONE, disp+27

LDD    #disp            ;load address of disp into D

JSR   display_string  ; Calling display_string to

BRA    loop

;**************************************************************

Conclusion:

- Concepts learnt:

  - In this lab, we gained a comprehensive understanding of memory management, subroutines, and stack operations in the context of the HC(S)12 microcontroller. This knowledge equips us with the ability to effectively utilize the stack to preserve register contents and facilitate communication with subroutines. Additionally, we explored the intricacies of banked memory and paging, crucial components of the HC(S)12 design. These concepts are foundational for more advanced programming endeavors and provide a solid framework for understanding microcontroller systems.

  - The usage of lookup tables for efficient LED control based on switch states was another valuable concept we learned. This approach allows for a streamlined and organized way to map input states to desired output patterns.

  - Finally, we also gained practical experience in interfacing with external components such as potentiometers and LCD displays. This skill set enables us to create more complex and interactive applications in the future.

- Concepts not learnt:

  - While the lab covered a wide range of topics, it would be beneficial to explore more advanced topics related to memory management and stack operations in future labs. Additionally, deeper insights into advanced subroutine usage and optimization techniques would be valuable for more complex projects.

- Difficulties encountered:

  - One of the challenges faced was ensuring the proper initialization and configuration of ports and registers to interface with external components. This required careful attention to detail and a solid understanding of the hardware specifications.

  - Additionally, working with lookup tables involved understanding how to efficiently map input states to desired output values. This required careful planning and organization of the lookup table data.

  - Debugging code for interactions with external hardware components can be challenging, especially when dealing with real-time input from switches and analog sensors. This required thorough testing and troubleshooting to ensure correct functionality.

Overall, this lab provided a valuable hands-on experience in working with memory management, subroutines, and stack operations, and it laid the groundwork for tackling more complex microcontroller projects in the future.