Ravi Mann

ECE 362

Post-Lab # 7

Introduction:

Introduction:

The purpose of this lab is to elevate your proficiency in assembly programming for the HCS12 microcontroller. By delving into more advanced concepts, we aim to equip you with the skills necessary to create intricate and well-structured programs.

Concepts Covered in the Lab:

- Implementing Control Structures:

  - This involves using constructs like loops (for, while) and conditional statements (if-else) to control program flow based on specific conditions.

- Basic Subroutines:

  - Create reusable blocks of code that can be called from different parts of a program. This promotes code modularity and efficiency.

- Stack Usage:

  - Understanding how the stack functions is essential for managing program execution and memory allocation. Also, gained insights into how to effectively use the stack.

- Software Delay Implementation:

  - Explored techniques to create delays in software, which can be crucial for synchronizing various parts of a program, especially in real-time applications.

- External Symbol Definition and Reference:

  - This concept involves compartmentalizing a project by separating different parts of the program into external symbols. This promotes modularity and simplifies project management.

Lab 6.1:

Objective/Purpose:

The objective of this lab is to enter, build, and execute specific programs. We will observe and record the value stored in accumulator A when the NOP instruction is reached. The programs will utilize for and while loops to traverse through an array.

Expected Results:

The program will initialize the data direction register of Port S for output. It will then iterate through the elements of the Array. If an element is non-zero, it will be output to Port S. Following the loop, there will be a delay of approximately 30 milliseconds. This entire process will continue indefinitely.

Explanation:

The program functions by first setting up Port S for output. It then enters a loop that iterates through the elements of the Array. For each element, it checks if it is non-zero. If so, it outputs the value to Port S. After completing the loop, a delay of about 30 milliseconds is introduced.

Results:

| Stack Address | Byte Data |
| --- | --- |
| $500 | -- |
| 4FC | C0 |
| 4FD | 11 |
| 4FC | 22 |
| 4FA | 33 |
| 4F8 | 44 |
| 4F9 | 44 |
| 4FA | 33 |
| 4FC | 22 |
| 4FE | C0 |

Observations:

The delay of approximately 30 milliseconds allows for a noticeable pause between iterations. The program effectively utilizes loops to iterate through the elements of the Array, providing a structured and controlled execution flow.

Code:

; Include derivative-specific definitions

INCLUDE 'derivative.inc'

; export symbols

XDEF Entry, _Startup, main

; we use export 'Entry' as symbol. This allows us to

; reference 'Entry' either in the linker .prm file

; or from C/C++ later on

XREF __SEG_END_SSTACK      ; symbol defined by the linker for the end of the stack

; code section

MyCode:     SECTION

main:

_Startup:

Entry:

LDS     #__SEG_END_SSTACK ; initialize the stack pointer

LDAA    #$11;

LDAB    #$22

LDX     #$3333

LDY     #$4444

JSR     StackRoutine

StackRoutine:

PSHA

PSHB

PSHX

PSHY

LDAA    #$0

LDAB    #$0

LDX     #$0

LDY     #$0

PULA

PULB

PULX

PULY

RTS             ; result in D

Lab 5.1.2:

Objective/Purpose:

The objective of this assembly code is to control the output of Port S based on the state of Port T. The program will read the state of Port T and select either Array1 or Array2 for output to Port S. It will achieve this through conditional branching and loops. Additionally, a delay subroutine is used to introduce a controlled pause in the program execution.

Explanation:

Data direction registers for Ports S and T are initialized. Reads Port T's current status and saves it in Val_T. Chooses either Array1 or Array2 for output to Port S based on the state of Port T. performs a check on each element in a loop across the chosen array. Outputs it to Port S if it is non-zero. brings in a delay of about 30 milliseconds. carries on endlessly in this manner.

Results:

The second part of the code likely follows a similar structure but with a different set of arrays and conditions based on Port T's state. This program will continuously run, selecting and outputting sequences based on Port T's state. The delay ensures controlled execution.

Code:

; Include derivative-specific definitions

INCLUDE 'derivative.inc'

; export symbols

XDEF Entry, _Startup, main

; we use export 'Entry' as symbol. This allows us to

; reference 'Entry' either in the linker .prm file

; or from C/C++ later on

XREF __SEG_END_SSTACK      ; symbol defined by the linker for the end of the stack

; variable/data section

Variables: Section

Constants: Section

Port_s  equ $248

Ddr_s   equ $24A

Port_t  equ $240

Mask    equ $FF

Mask1    equ $01

MyCode: SECTION

main:

_Startup:

Entry:

start:

BSET    Ddr_s, Mask  ; set all the leds for port s

loop_0high:  BRCLR   Port_t, Mask1, loop_0high ;branch if switch 1 is low

loop_0low:  BRSET   Port_t, Mask1, loop_0low  ;branch if switch 1 is high

loop_0high2:  BRCLR   Port_t, Mask1, loop_0high2 ;branch if switch 1 is low

BSET    Port_s, #%01000000

NOP

Lab 6.2:

Objective/Purpose:

The objective of this program is to create a lookup table-based system for controlling LEDs based on the state of switches. It aims to read the switch input continuously, use a lookup table to map each switch state to a specific LED output value, and display the corresponding LED pattern on the LEDs connected to the microcontroller.

Explanation:

This program continuously monitors the state of switches connected to Port T. It uses a lookup table to determine the corresponding LED pattern and displays it on Port S. The program runs in an infinite loop, providing real-time interaction between switches and LEDs.

Results:

The program will initialize Port S for output. It will then continuously monitor the state of the switches connected to Port T. Whenever the switch state changes, the program will use the lookup table to determine the corresponding LED pattern and display it on Port S. The program will run in an infinite loop, providing real-time interaction between switches and LEDs.

Code:

XDEF  SendMessageInfo

XREF  SendsChr

SendMessageInfo:

LEAX  3, SP     ; Array

LDY   5, SP     ; Counter

Loop:

LDAA  1, X+

PSHA

CALL  SendsChr

PULA

LEAS  -1, SP

DEY

CPY   #0

BNE   Loop

RTC

Lab 6.3:

Objective/Purpose:

The objective of this lab is to develop an assembly program capable of searching for a specific value, represented by $dd, within a predefined lookup table. The program is designed to determine the index of this value within the table. If a match is found, the program should return the index value in the accumulator (A). Conversely, if no match is found, it should load the value $FF into the accumulator.

Explanation:

The program employs a straightforward linear search as its methodology. It progressively checks each element of the lookup table until it finds a match or reaches the end of the table. The loop iterates again, increasing the index (B) each time. This index is used to keep track of a place within the Lookup table. The software effectively controls the search process and provides detailed information on whether a match was found. The software is structured with the appropriate labels and comments, which makes it easier for developers to read and understand. With the use of conditional branches (BEQ), which are crucial for controlling program flow, the program may make choices based on the results of the comparisons.

Results:

The program will locate the index of the value stored in Val and return it to the accumulator (A) if the value is located in the Lookup table. If the lookup table does not include the value contained in Val, the program will search the whole table in vain. The accumulator (A) will be loaded with $FF in this situation.

Code:

;**************************************************************

;* This stationery serves as the framework for a              *

;* user application. For a more comprehensive program that    *

;* demonstrates the more advanced functionality of this       *

;* processor, please see the demonstration applications       *

;* located in the examples subdirectory of the                *

;* Freescale CodeWarrior for the HC12 Program directory       *

;**************************************************************

; Include derivative-specific definitions

INCLUDE 'derivative.inc'

; export symbols

XDEF Entry, _Startup, main

; we use export 'Entry' as symbol. This allows us to

; reference 'Entry' either in the linker .prm file

; or from C/C++ later on

XDEF Port_u, Entry, _Startup, main, ROW_SCAN, LU_PORT_U, PD_BUTTONS, KEY_PRESSED,COUNT,Port_t,T_ON,CRGFLG

XREF  __SEG_END_SSTACK, KEYPAD,RTI_ISR  ; symbol defined by the linker for the end of the stack

; variable/data section

MY_VAR: SECTION

; Insert here your data definition.

COUNT: DS.B 1

TEMP: DS.B 1

LU_PORT_U: DS.B  1

KEY_PRESSED: DS.B 1

T_ON: DS.B 1 ;KEY_PREESED

T_OFF: DS.B 1;SET TO 15 DECIMAL OR $0F

;Constants

MY_CONST: SECTION

Port_t: EQU $240 ; SWITCH./ DC_MOTOR

Port_t_ddr: EQU $242 ;BIT3 DC MOTOR

Port_u:      EQU $268

Port_u_ddr:  EQU $26A

Port_u_psr:  EQU $26D

Port_u_pder: EQU $26C

Port_p: equ $258

Port_p_ddr equ $25A

Port_s: EQU $248

Port_s_ddr: EQU $24A

ROW_SCAN:   DC.B  $80, $40, $20, $10, $00

PD_BUTTONS:  DC.B  $14, $88, $84, $82, $48, $44, $42, $28, $24, $22, $18, $12, $81, $41, $21, $11

IER: EQU $0038 ;INTERRUPT ENEBALE REGISTER

ICR: EQU $003B ;INTERRUPT CONTROL REGISTER

Array DC.B $0A,$12,$14,$0C,$0

; code section

MyCode:     SECTION

main:

_Startup:

Entry:

LDS   #__SEG_END_SSTACK ; initialize the stack pointer

BSET  Port_u_ddr, #$F0

BSET  Port_u_psr, #$0F

BSET  Port_u_pder, #$0F

BSET  Port_t_ddr, #$08  ;BSET  Port_t, #$08 ; switch 4 is up DC can now spin / possible set to brset later

;to stop BCLR PORT_T, #$00

BSET  Port_s_ddr, #$FF

BSET  Port_t_ddr, #$08

CLI                     ;clear I flag in ccr

BSET  IER,  #$80        ;Sets bit 7 of interrupt enebel register

BSET  ICR,  #$60        ;4,000 ms delay

Check_state:

BRSET   Port_t, #$01, State_B_start             ; branch to Clockwise if bit 1 is 1

BRA     State_A_start                           ; branch back to Loop1 because both bits are 00

State_A_start:

BCLR   Port_p_ddr,#$FF                          ;maybe check again if switch is still 1

BRA    State_A

LDAA   #$3                                      ;used for testing

STAA   Port_s

BRA    Check_state

State_B_start:

BCLR   Port_t_ddr,#$08                           ;DC Motor not spining

BRA    State_B

LDAA   #$2                                       ;used for testing

STAA   Port_s

BRA    Check_state

State_A:

JSR     KEYPAD

LDAB    KEY_PRESSED

STAB    T_ON

BRA     Check_state

BRA     State_A

State_B:

LDAA    #%01111000

STAA    Port_p_ddr

Load:       LDX     #Array

Loop:       LDAA    1,x+

CMPA    #0

BEQ     Load

STAA    Port_p

BRA     Delay

Delay:      LDY     #$4000

Delay_Loop: DEY

BNE     Delay_Loop

BRA     Check_state

BRA     Loop

Done:

NOP

XDEF  KEYPAD

XREF  Port_u, ROW_SCAN, KEY_PRESSED , PRESSED_OR_NOT_PRESSED ,LU_PORT_U, PD_BUTTONS

KEYPAD:         PSHB

PSHA

LOAD_FIRST_ROW: LDX   #ROW_SCAN       ; load x with first row array value

LOAD_NEXT_ROW:  LDAB  1,X+            ; load row into b, and set x to next row

BEQ   Not_pressed             ; if checked last row, go back to first

STAB  Port_u          ; send the row array value to port u

JSR   DELAY_ENTRY     ; delay for 1ms

BRCLR Port_u, #$0F, LOAD_NEXT_ROW ; if no button pressed then load the next row. will continue on if a button has been pressed

LDAA  Port_u          ; load port u into a

STAA  LU_PORT_U       ; save port u value for later. use with lu table

RELEASED:       LDX   #PD_BUTTONS    ; address of first pull down button

LDAA  #00            ; initialize counter

LU_TABLE_LOOP:  LDAB  1,X+           ; load address of pull down button into x and go to next button

CMPB  LU_PORT_U      ; if match is found

BEQ   MATCH_FOUND

INCA

BRA LU_TABLE_LOOP

Not_pressed:    PULA

PULB

RTS

MATCH_FOUND:    STAA  KEY_PRESSED    ; store the counter to key_pressed

PULA

PULB

RTS                  ; go back to main loop

DELAY_ENTRY:  LDY #3000

DELAY_LOOP    DEY

BNE DELAY_LOOP

RTS

XDEF RTI_ISR

XREF COUNT,Port_t,T_ON,CRGFLG

RTI_ISR:

LDAA COUNT

BRA REAL_TIME_INTERUPTS

REAL_TIME_INTERUPTS:

INCA;Increment Count

STAA COUNT

CMPA T_ON ;A-MEM

BMI COUNT_LESS_THAN_T_ON

BRA COUNT_GREATER_THAN_T_ON

COUNT_LESS_THAN_T_ON:

BSET Port_t,#$08

BSET CRGFLG,#$80;clear RTIF

RTI;Return from interupt

COUNT_GREATER_THAN_T_ON

CMPA #$0F

BMI LESS_THAN_15

BRA GREATER_THAN_15

LESS_THAN_15:

BCLR Port_t,#$08

BSET CRGFLG,#$80;clear RTIF

RTI;Return from interupt

GREATER_THAN_15:

LDAA #$0

STAA COUNT

BSET CRGFLG,#$80;clear RTIF

RTI;Return from interupt