Ravi Mann

ECE 362

Pre-Lab #6

Introduction:

Objective & Purpose:

This lab's main goal is to provide me the information and abilities I need to use interruptions efficiently. In embedded systems, interruptions are essential because they let the microcontroller react quickly to important events without continuously polling the system. This lab will teach me how to include interruptions into my programs so that I may control several devices at once.

Lab 7.1:

Objective/Purpose:

The objective of this experiment is to develop an assembly program that utilizes Pulse Width Modulation (PWM) to regulate the speed of a DC motor connected to bit 3 of Port T on the I/O board. By the end of this lab, you should have a functional program that effectively controls the DC motor's speed.

Expected Results:

The DC motor should start and run at a variable speed, controlled by the PWM signal. Switch 4 must be in the "up" position for the DC motor to spin. Setting bit 3 of Port T at address $240 should initiate the motor, while clearing bit 3 should bring it to a halt.

Code:

Entry:

LDS #__SEG_END_SSTACK ; initialize the stack pointer

LDAA $11;

LDAB $22

LDX $3333

LDY $4444

JSR StackRoutine

StackRoutine:

PSHA

PSHB

PSHX

PSHY

LDAA	#$0

LDAB	#$0

LDX	#$0

LDY	#$0

PULA

PULB

PULX

PULY

RTS

Lab 6.2:

Objective/Purpose:

The objective of this experiment is to develop an assembly program that utilizes Pulse Width Modulation (PWM) to regulate the speed of a DC motor connected to bit 3 of Port T on the I/O board. Unlike previous experiments, instead of using delay loops and polling, Real Time Interrupts (RTIs) will be employed to implement the PWM strategy. The RTI interval will be set to 4ms.

Expected Results:

The DC motor should start and run at a variable speed, controlled by the PWM signal. The PWM signal will be generated using Real Time Interrupts (RTIs) at an interval of 4ms. The unaltered keypad subroutine from Lab 5.2 should be integrated into the main program.

Lab 6.3:

Objective/Purpose:

This exercise aims to get me ready for my final project by creating a state machine in the main.asm file. Flags will be used by this state machine to regulate the tasks associated with the present state of the system. Use of Real Time Interrupts (RTI) is to regulate the timing of events, such LEDs blinking after a delay. Setting flags in the RTI will cause these events to occur, and the state machine will recognize and handle them. This method is essential since RTI happens often and is limited to short, urgent activities.

Expected Results:

State A and State B represent the two states that make up the system. With keypad input and PWM controlling the DC motor, State A should see board behavior like to Lab 7.2. In State B, the stepper motor should spin in one direction, similar to Lab 4.3. It should be possible to transition between States A and B. The functionalities linked to the other state ought to be deactivated while the active state is in place.