Ravi Mann

ECE 362

Post-Lab # 7

Introduction:

This lab focused on implementing interruptions in microcontrollers. Throughout this lab, the primary aim is to understand the essence of interrupts—signals triggered by hardware or software events that redirect a microcontroller's operations to a designated Interrupt Service Routine (ISR). The key objectives include understanding how to manage multiple time and event-sensitive tasks using interrupts and modifying linker parameter files to accommodate diverse entries in the vector table.

Lab 7.1:

Objective/Purpose:

The code seems to facilitate keypad input processing to control a DC motor through PORT_T based on the keys pressed. It includes a keypad reading subroutine and sequences to handle motor control based on input.

Expected Results:

The code appears to handle keypad input efficiently and control a DC motor spin through PORT_T. It implements sequences to manage Port_u_array values and delays, aiming to control the motor based on these inputs. Observing physical output (motor spin or LED control) is essential to validate the code's functionality and confirm its alignment with the intended purpose.

Explanation:

Initializes stack pointer, control bits for various ports (PORT_T and PORT_U), and sets up necessary variables.

Reads inputs from PORT_U (keypad) and processes the key presses. Sets motor spin through PORT_T based on keypad inputs. Loops through an array, uses Start_Dealy subroutine for delay, and processes the PORT_U array.

Code:

;**************************************************************

;* This stationery serves as the framework for a              *

;* user application. For a more comprehensive program that    *

;* demonstrates the more advanced functionality of this       *

;* processor, please see the demonstration applications       *

;* located in the examples subdirectory of the                *

;* Freescale CodeWarrior for the HC12 Program directory       *

;**************************************************************

; Include derivative-specific definitions

INCLUDE 'derivative.inc'

; export symbols

XDEF Entry, _Startup, main

; we use export 'Entry' as symbol. This allows us to

; reference 'Entry' either in the linker .prm file

; or from C/C++ later on

XDEF PORT_U, Entry, _Startup, main, Row_scan, LU_PORT_U, PD_BUTTONS, Key_pressed

XREF __SEG_END_SSTACK, Read_PortU      ; symbol defined by the linker for the end of the stack

; variable/data section

MY_VAR: SECTION

; Insert here your data definition.

LU_PORT_U:  DS.B  1

Key_pressed:DS.B 1

T_on:       DS.B 1 ;KEY_PREESED

T_off:      DS.B 1;SET TO 15 DECIMAL OR $0F

;Constants

MY_CONST: SECTION

PORT_T:       EQU $240 ; SWITCH./ DC_MOTOR

PORT_T_DDR:   EQU $242 ;BIT3 DC MOTOR

PORT_U:       EQU $268

PORT_U_DDR:   EQU $26A

PORT_U_PSR:   EQU $26D

PORT_U_PDER:  EQU $26C

Row_scan:     DC.B  $80, $40, $20, $10, $00

PD_BUTTONS:   DC.B  $14, $88, $84, $82, $48, $44, $42, $28, $24, $22, $18, $12, $81, $41, $21, $11

; code section

MyCode:     SECTION

main:

_Startup:

Entry:

LDS  #__SEG_END_SSTACK     ; initialize the stack pointer

; Setting control bits for various ports and switches

BSET  $24A, #$FF       ; Set control bits

BSET  PORT_U_DDR, #$F0 ; Set data direction for PORT_U

BSET  PORT_U_PSR, #$0F ; Set port pins for PORT_U

BSET  PORT_U_PDER, #$0F ; Set port data enable register for PORT_U

BSET  PORT_T_DDR, #$08 ; Set data direction for PORT_T

BSET  PORT_T, #$08     ; Activate switch 4 (DC can now spin)

Read_Keypad:

JSR   Read_PortU  ; Jump to subroutine to read PORT_U for keypad input

KEY_IS_PRESSED:

LDAB  Key_pressed  ; Load value of Key_pressed

STAB  T_on         ; Store in T_on

LDAA  #$0F         ; Load constant value

SBA                ; Subtract A from B

STAA  T_off        ; Store result in T_off

BRA   T_ON_CHECK   ; Branch to T_ON_CHECK

T_ON_CHECK:

LDAA  T_on         ; Load value of T_on

CMPA  #$01         ; Compare with constant 1

BMI   T_OFF_CHECK  ; Branch if minus to T_OFF_CHECK

BRA   T_ON_NOT_ZERO; Branch if not minus to T_ON_NOT_ZERO

T_ON_NOT_ZERO:

BSET   PORT_T,#$08 ; Set bit for PORT_T

LDY    #4000       ; Load delay count for PWM start

PWM_DELAY1:

DEY               ; Decrement delay count

BNE    PWM_DELAY1 ; Loop until delay count is zero

DEC    T_on       ; Decrement T_on

BRA    T_ON_CHECK ; Branch back to T_ON_CHECK

T_OFF_CHECK:

LDAB   T_off      ; Load value of T_off

CMPB   #01        ; Compare with constant 1

BMI    Read_Keypad; Branch if minus to Read_Keypad

BRA    T_OFF_NOT_ZERO; Branch if not minus to T_OFF_NOT_ZERO

T_OFF_NOT_ZERO:

BCLR    PORT_T,#$08 ; Clear bit for PORT_T

LDY     #4000      ; Load delay count for PWM start

PWM_DELAY2:

DEY               ; Decrement delay count

BNE     PWM_DELAY2; Loop until delay count is zero

DEC     T_off     ; Decrement T_off

BRA     T_OFF_CHECK; Branch back to T_OFF_CHECK

XDEF  KEYPAD

XREF  PORT_U, ROW_SCAN, KEY_PRESSED , PRESSED_OR_NOT_PRESSED ,LU_PORT_U, PD_BUTTONS

XDEF Sequence

XREF Port_u, Start_Dealy, Count, Current, Temp, Port_u_array, Read_array

Sequence:

LDX	  #Port_u_array	  	; load address of array into X

Next:         LDAA  1,X+				; Load value of X into A

; If end of the array is reached then reload the array back into X

BEQ   Sequence

STAA	Port_u		    ; Load value of array into Port_u

; Increment the register X

JSR	  Start_Dealy		; Start delay of 1 millisecond

Read_portU:		LDAA	Port_u        ; Read value of Port_u into A

STAA  Current			  ; Save the value of Port_u for later use

STAA  Temp

BRCLR Temp, #$0F, Next ;Not pressed then load next row

;ANDA	#$0F		     	; Ignore the 3 and 4 bytes

;CMPA  #$0F				  ; Test if A = $0F / no key is pressed

;BEQ 	Next          ; Zero then branch to next sequence

;BRA   Key_pressed		; Branch to Key_pressed

Key_pressed:

LDAA  Port_u			   ; Read value of Port_u into A

STAA  Temp

BRCLR Temp, #$0F, Not_pressed ; Key not pressed and branch

;ANDA  #$0F			    ; Ignore the 3 and 4 bytes

;CMPA  #$0F			    ; Test if A = $0F / no key is pressed

;BEQ   Not_pressed		; Zero then branch to key Not_pressed

BRA   Key_pressed		; Branch to back to Key_pressed

Not_pressed:  LDY		  #Read_array	; Load address of Lookup Table into Y

LDAA     #0   ; Load A with Count value

Loop:         LDAB    1, Y+   ; Load value from X+1 into B and X=X+1

CMPB    #00	    ; Check if Look up value is at the end of the array

BEQ     Not_pressed     ;

CMPB		Current ; Get Lookup Value

BEQ     Match

INCA            ; Increment A

BRA		  Loop		; Branch to loop

Match:

STAA    Current

RTS

Lab 7.2:

Objective/Purpose:

The code seems to manage a keypad input interface that interacts with different registers to control motor operation and LED blinking patterns. It utilizes interrupts to handle real-time operations.

Explanation:

KEYPAD function reads keypad inputs, handles button presses, and stores the pressed key into KEY_PRESSED. Sets up ports (Port_u, Port_t), initializes their directions, and starts the DC motor.  RTI_ISR manages state transitions, LED blinking patterns, and increments counters.

Results:

The code appears to initialize ports and handle keypad inputs to control a DC motor. It implements real-time interrupts to handle LED blinking patterns and manage the state transitions based on the motor's operation and keypad inputs. No specific observations on the physical output can be made without knowing the context or observing the actual behavior of the motor and LEDs connected to these ports.

Code:

XDEF  KEYPAD

XREF  PORT_U, ROW_SCAN, KEY_PRESSED , PRESSED_OR_NOT_PRESSED ,LU_PORT_U, PD_BUTTONS

KEYPAD:         PSHB

PSHA

LOAD_FIRST_ROW: LDX   #ROW_SCAN      ; load x with first row array value

LOAD_NEXT_ROW:  LDAB  1,X+          ; load row into b, and set x to next row

;CMPB  #$0

BEQ   PWM ; if checked last row, go back to first

STAB  PORT_U        ; send the row array value to port u

JSR   DELAY_ENTRY   ; delay for 1ms

BRCLR PORT_U, #$0F, LOAD_NEXT_ROW ; if no button pressed then load the next row. will continue on if a button has been pressed

LDAA  PORT_U        ; load port u into a

STAA  LU_PORT_U     ; save port u value for later. use with lu table

;  MOVB LU_PORT_U,$248

;CHECK_RELEASE:  LDAB  PORT_U        ; read in port u again

;                STAB  TEMP_PORT_U   ; store port u to temp location

;               BRCLR TEMP_PORT_U, #$0F, RELEASED

;                BRA   CHECK_RELEASE

;

RELEASED:       LDX   #PD_BUTTONS    ; address of first pull down button

LDAA  #00            ; initialize counter

LU_TABLE_LOOP:  LDAB  1,X+           ; load address of pull down button into x and go to next button

CMPB  LU_PORT_U      ; if match is found

BEQ   MATCH_FOUND

INCA

bra LU_TABLE_LOOP

PWM:    PULA

PULB

RTS

; BRA  PRESSED_OR_NOT_PRESSED  ; if a match is not found branch to PWM ROUTINE

MATCH_FOUND:    STAA  KEY_PRESSED    ; store the counter to key_pressed

PULA

PULB

RTS                  ; go back to main loop

DELAY_ENTRY:  LDY #3000

DELAY_LOOP    DEY

BNE DELAY_LOOP

RTS

XDEF RTI_ISR,       ; Define RTI_ISR for external reference

XREF COUNT,PORT_T,T_ON,CRGFLG,PORT_P_DDR, PORT_T_DDR,PORT_S, STATE_FLAG, STEEPER_COUNT,RTI_COUNT,LED_COUNT_STATE_A,LED_COUNT_STATE_B

RTI_ISR:

; LDAA #$0;Set Count = 0

; STAA COUNT

RTI_CHECK_STATE:

BRSET   PORT_T, #$01, SET_LOW_FLAG        ; branch to Clockwise if bit 1 is 1

BRA     SET_HIGH_FLAG

; branch back to Loop1 because both bits are 00

SET_LOW_FLAG:

LDAA #0

STAA STATE_FLAG

BRA LED_A_INSTANCE_CHECK

SET_HIGH_FLAG:

LDAA #1

STAA STATE_FLAG

BRA LED_B_INSTANCE_CHECK

;

LED_A_INSTANCE_CHECK:

LDAA LED_COUNT_STATE_A

INCA

CMPA #15

BEQ LED_A_DELAY

BRA NOT_YET_LED_A

NOT_YET_LED_A:

STAA LED_COUNT_STATE_A

BSET CRGFLG,#$80;clear RTIF

RTI;Return from interrupt

LED_A_DELAY:

LDAB PORT_S

EORB #$FF

STAB PORT_S

LDAA #0

STAA LED_COUNT_STATE_A

BRA REAL_TIME_INTERUPTS

;

REAL_TIME_INTERUPTS:

LDAA COUNT

INCA;Increment Count

STAA COUNT

CMPA T_ON ;A-MEM

Bmi COUNT_LESS_THAN_T_ON

BRA COUNT_GREATER_THAN_T_ON

COUNT_LESS_THAN_T_ON:

BSET PORT_T,#$08

BSET CRGFLG,#$80;clear RTIF

RTI;Return from interrupt

COUNT_GREATER_THAN_T_ON

CMPA #$0F

BMI LESS_THAN_15

BRA GREATER_THAN_15

LESS_THAN_15:

BCLR PORT_T,#$08

BSET CRGFLG,#$80;clear RTIF

RTI;Return from interrupt

GREATER_THAN_15:

LDAA #$0

STAA COUNT

BSET CRGFLG,#$80;clear RTIF

RTI;Return from interrupt

LED_B_INSTANCE_CHECK:

LDAA LED_COUNT_STATE_B

INCA

CMPA #7

BEQ LED_B_DELAY

BRA NOT_YET_LED_B

NOT_YET_LED_B:

STAA LED_COUNT_STATE_B

BSET CRGFLG,#$80;clear RTIF

RTI;Return from interrupt

LED_B_DELAY:

LDAB PORT_S

EORB #$FF

STAB PORT_S

LDAA #0

STAA LED_COUNT_STATE_B

BRA RTI_INSTANCE_CHECK

;WE NEED TO RUN THE RTI AT LEAST 5-8 INSTANCES TO IMPLEMENT STAE_B

;8*4000 = 32,000 MS

RTI_INSTANCE_CHECK:

LDAA RTI_COUNT

INCA

CMPA #7

BEQ STEEPER_MOTOR_DELAY

BRA NOT_8TH_INSTANCE

NOT_8TH_INSTANCE:

STAA RTI_COUNT

BSET CRGFLG,#$80;clear RTIF

RTI;Return from interrupt

STEEPER_MOTOR_DELAY:

LDAA #$0

STAA RTI_COUNT

LDAA STEEPER_COUNT

INCA

STAA STEEPER_COUNT

CMPA #$4

BEQ RESTART

BRA JUMP_OUT

RESTART:

LDAA #0

STAA STEEPER_COUNT

JUMP_OUT:

BSET CRGFLG,#$80;clear RTIF

RTI

;**************************************************************

;* This stationery serves as the framework for a              *

;* user application. For a more comprehensive program that    *

;* demonstrates the more advanced functionality of this       *

;* processor, please see the demonstration applications       *

;* located in the examples subdirectory of the                *

;* Freescale CodeWarrior for the HC12 Program directory       *

;**************************************************************

; Include derivative-specific definitions

INCLUDE 'derivative.inc'

; export symbols

XDEF Entry, _Startup, main

; we use export 'Entry' as symbol. This allows us to

; reference 'Entry' either in the linker .prm file

; or from C/C++ later on

XDEF Port_u, Entry, _Startup, main, ROW_SCAN, LU_PORT_U, PD_BUTTONS, KEY_PRESSED,COUNT,Port_t,T_ON,CRGFLG;PRESSED_OR_NOT_PRESSED,

XREF  __SEG_END_SSTACK, key,RTI_ISR  ; symbol defined by the linker for the end of the stack

;Constants

MY_CONST: SECTION

Port_t: EQU $240 ; SWITCH./ DC_MOTOR

Port_t_ddr: EQU $242 ;BIT3 DC MOTOR

Port_u:      EQU $268

Port_u_ddr:  EQU $26A

Port_u_psr:  EQU $26D

Port_u_pder: EQU $26C

ROW_SCAN:   DC.B  $80, $40, $20, $10, $00

PD_BUTTONS:  DC.B  $14, $88, $84, $82, $48, $44, $42, $28, $24, $22, $18, $12, $81, $41, $21, $11

IER: EQU $0038 ;INTERRUPT ENABLE REGISTER

ICR: EQU $003B ;INTERRUPT CONTROL REGISTER

; variable/data section

MY_VAR: SECTION

; Insert here your data definition.

COUNT: DS.B 1

TEMP: DS.B 1

LU_PORT_U: DS.B  1

KEY_PRESSED: DS.B 1

T_ON: DS.B 1 ;KEY_PRESSED

T_OFF: DS.B 1;SET TO 15 DECIMAL OR $0F

; code section

MyCode:     SECTION

main:

_Startup:

Entry:

LDS   #__SEG_END_SSTACK   ; initialize the stack pointer

BSET  $24A, #$FF

BSET  Port_u_ddr, #$F0

BSET  Port_u_psr, #$0F

BSET  Port_u_pder, #$0F

BSET  Port_t_ddr, #$08  ;BSET  Port_t, #$08 ; switch 4 is up DC can now spin / possible set to brset later

BSET  Port_t, #$08        ; switch 4 is up DC can now spin / possible set to brset later

;to stop BCLR PORT_T, #$00

CLI                        ;clear I flag

Lab 7.3:

Objective/Purpose:

The objective of this lab is to develop an assembly program capable of controlling the motor control based on the inputs from keypad and real tiem interrupts. It also utilizes a state flag to transition between two states to control the motor and stepper.

Explanation:

Initializes ports (PORT_U, PORT_T, etc.), stack pointers, and interrupt registers. The KEYPAD function reads inputs from a keypad, stores pressed keys in KEY_PRESSED. Controls the motor when in State A, setting the motor speed based on the keypad input. Stops the motor in State B and manages the PORT_P values for motor control.

Results:

The code appears designed to manage a DC motor based on keypad inputs, transitioning between two states for motor control. The actual outcomes would be observed in the physical behavior of the motor when interfaced with this code. Observations can include motor speed changes based on keypad input, the effect of state transitions on motor operation, and the delay introduced by real-time interrupts.

Code:

;**************************************************************

;* This stationery serves as the framework for a              *

;* user application. For a more comprehensive program that    *

;* demonstrates the more advanced functionality of this       *

;* processor, please see the demonstration applications       *

;* located in the examples subdirectory of the                *

;* Freescale CodeWarrior for the HC12 Program directory       *

;**************************************************************

; Include derivative-specific definitions

INCLUDE 'derivative.inc'

; export symbols

XDEF Entry, _Startup, main

; we use export 'Entry' as symbol. This allows us to

; reference 'Entry' either in the linker .prm file

; or from C/C++ later on

XDEF PORT_U, Entry, _Startup, main, ROW_SCAN, LU_PORT_U, PD_BUTTONS, KEY_PRESSED,COUNT,PORT_T,T_ON,CRGFLG,PORT_P_DDR, PORT_T_DDR,PORT_S,STATE_FLAG,STEEPER_COUNT,RTI_COUNT,LED_COUNT_STATE_A,LED_COUNT_STATE_B

XREF  __SEG_END_SSTACK, KEYPAD,RTI_ISR, CHECK_STATE ; symbol defined by the linker for the end of the stack

; variable/data section

MY_VAR: SECTION

; Insert here your data definition.

COUNT: DS.B 1

TEMP: DS.B 1

LU_PORT_U: DS.B  1

KEY_PRESSED: DS.B 1

T_ON: DS.B 1 ;KEY_PREESED

T_OFF: DS.B 1;SET TO 15 DECIMAL OR $0F

STATE_FLAG: DS.B 1

STEEPER_COUNT: DS.B 1

LED_COUNT_STATE_A: DS.B 1

LED_COUNT_STATE_B: DS.B 1

RTI_COUNT: DS.B 1;CHECKS HOW MANY TIMES WE ENTER THE RTI

;Constants

MY_CONST: SECTION

PORT_T: EQU $240 ; SWITCH./ DC_MOTOR

PORT_T_DDR: EQU $242 ;BIT3 DC MOTOR

PORT_U:      EQU $268

PORT_U_DDR:  EQU $26A

PORT_U_PSR:  EQU $26D

PORT_U_PDER: EQU $26C

ROW_SCAN:   DC.B  $80, $40, $20, $10, $00

PD_BUTTONS:  DC.B  $14, $88, $84, $82, $48, $44, $42, $28, $24, $22, $18, $12, $81, $41, $21, $11

IER: EQU $0038 ;INTERRUPT ENEBALE REGISTER

ICR: EQU $003B ;INTERRUPT CONTROL REGISTER

PORT_P: equ $258

PORT_P_DDR equ $25A

ARRAY DC.B $0A,$12,$14,$0C

PORT_S: EQU $248

PORT_S_DDR: EQU $24A

; code section

MyCode:     SECTION

main:

_Startup:

Entry:

LDS   #__SEG_END_SSTACK     ; initialize the stack pointer

BSET  PORT_U_DDR, #$F0

BSET  PORT_U_PSR, #$0F

BSET  PORT_U_PDER, #$0F

BSET  PORT_T_DDR, #$08

;BSET  PORT_T, #$08 ; switch 4 is up DC can now spin / possible set to brset later

;to stop BCLR PORT_T, #$00

BSET PORT_S_DDR,#$FF

BSET PORT_T_DDR,#$08

CLI;clear I flag in ccr

BSET IER,#$80;Sets bit 7 of interrupt enable register

BSET ICR,#$60 ;4,000 ms delay

BSET PORT_P_DDR, #$1E

MAIN_CHECK_STATE:

BRSET   STATE_FLAG, #$01,STATE_A_TRANSITION         ; branch to Clockwise if bit 1 is 1

BRA STATE_B_TRANSITION

STATE_A_TRANSITION:

BRA STATE_A

STATE_B_TRANSITION:

BRA STATE_B

STATE_A: ;0

KEY_LOOP:

JSR  KEYPAD  			;Read the keypad

LDAB KEY_PRESSED		; Load key pressed into B

STAB T_ON				; Store the motor speed value onto T_on

LDAA STATE_FLAG

CMPA #1				; Check if we are in state A or B

BMI STATE_A

BRA STATE_B

STATE_B:  ;1

BCLR PORT_T,#$08			;DC Motor not spinning

LDAA #$1e

STAA PORT_P_DDR

START:

LDX #ARRAY

LDAB STEEPER_COUNT		;add the value of current time into b

ABX					; add the time value to X

LDAA 0,X				; load the current value of array into Port P

STAA  PORT_P

BRA MAIN_CHECK_STATE

XDEF  KEYPAD

XREF  Port_u, ROW_SCAN, KEY_PRESSED , PRESSED_OR_NOT_PRESSED ,LU_PORT_U, PD_BUTTONS

KEYPAD:         PSHB

PSHA

LOAD_FIRST_ROW: LDX   #ROW_SCAN       ; load x with first row array value

LOAD_NEXT_ROW:  LDAB  1,X+            ; load row into b, and set x to next row

BEQ   Not_pressed             ; if checked last row, go back to first

STAB  Port_u          ; send the row array value to port u

JSR   DELAY_ENTRY     ; delay for 1ms

BRCLR Port_u, #$0F, LOAD_NEXT_ROW ; if no button pressed then load the next row. will continue on if a button has been pressed

LDAA  Port_u          ; load port u into a

STAA  LU_PORT_U       ; save port u value for later. use with lu table

RELEASED:       LDX   #PD_BUTTONS    ; address of first pull down button

LDAA  #00            ; initialize counter

LU_TABLE_LOOP:  LDAB  1,X+           ; load address of pull down button into x and go to next button

CMPB  LU_PORT_U      ; if match is found

BEQ   MATCH_FOUND

INCA

BRA LU_TABLE_LOOP

Not_pressed:    PULA

PULB

RTS

MATCH_FOUND:    STAA  KEY_PRESSED    ; store the counter to key_pressed

PULA

PULB

RTS                  ; go back to main loop

DELAY_ENTRY:  LDY #3000

DELAY_LOOP    DEY

BNE DELAY_LOOP

RTS

XDEF RTI_ISR,

XREF COUNT,PORT_T,T_ON,CRGFLG,PORT_P_DDR, PORT_T_DDR,PORT_S, STATE_FLAG, STEEPER_COUNT,RTI_COUNT

RTI_ISR:

; LDAA #$0;Set Count = 0

; STAA COUNT

RTI_CHECK_STATE:

BRSET   PORT_T, #$01, SET_LOW_FLAG        ; branch to Clockwise if bit 1 is 1

BRA     SET_HIGH_FLAG

; branch back to Loop1 because both bits are 00

SET_LOW_FLAG:

BCLE	STATE

STAA STATE_FLAG

BRA REAL_TIME_INTERUPTS

SET_HIGH_FLAG:

LDAA #1

STAA STATE_FLAG

BRA RTI_INSTANCE_CHECK

REAL_TIME_INTERUPTS:

LDAA COUNT

INCA;Increment Count

STAA COUNT

CMPA T_ON ;A-MEM

Bmi COUNT_LESS_THAN_T_ON

BRA COUNT_GREATER_THAN_T_ON

COUNT_LESS_THAN_T_ON:

BSET PORT_T,#$08

BSET CRGFLG,#$80;clear RTIF

RTI;Return from interupt

COUNT_GREATER_THAN_T_ON

CMPA #$0F

BMI LESS_THAN_15

BRA GREATER_THAN_15

LESS_THAN_15:

BCLR PORT_T,#$08

BSET CRGFLG,#$80;clear RTIF

RTI;Return from interupt

GREATER_THAN_15:

LDAA #$0

STAA COUNT

BSET CRGFLG,#$80;clear RTIF

RTI;Return from interupt

;WE NEED TO RUN THE RTI AT LEAST 5-8 INSTANCES TO IMPLEMENT STAE_B

;8*4000 = 32,000 MS

RTI_INSTANCE_CHECK:

LDAA RTI_COUNT

INCA

CMPA #2

BEQ STEEPER_MOTOR_DELAY

BRA NOT_8TH_INSTANCE

NOT_8TH_INSTANCE:

STAA RTI_COUNT

BSET CRGFLG,#$80;clear RTIF

RTI;Return from interupt

STEEPER_MOTOR_DELAY:

LDAA #$0

STAA RTI_COUNT

LDAA STEEPER_COUNT

INCA

STAA STEEPER_COUNT

CMPA #$4

BEQ RESTART

BRA JUMP_OUT

RESTART:

LDAA #0

STAA STEEPER_COUNT

JUMP_OUT:

BSET CRGFLG,#$80 		;clear RTIF

RTI