Ravi Mann

ECE 362

Post-Lab # 5

Introduction:

Introduction:

The objective of this lab is to employ logic, bit, and shift instructions to demonstrate various applications for I/O peripherals. Debouncing inputs from mechanical switches, Boolean logic rules, shift and rotate rules, bit test and manipulation rules, bit condition branch rules, and switch debouncing were among the ideas studied.

Lab 5.1.1:

Objective/Purpose:

The objective of this lab is to enter, build, and execute specific programs. To set or clear bit values, logical operations like AND and OR are implemented.

Explanation:

The code manipulates specific bits of certain memory-mapped registers, using logic instructions.

Results:

The program reads VAR_1, clears the sixth bit, writes the value back into VAR_1, sets the fourth bit and saves it back into VAR_1, and then waits for bit 0 (switch 1), sending VAR_1 to the LEDs when switch 2 is low.

Code:

INCLUDE 'derivative.inc'

; export symbols

XDEF Entry, _Startup, main

; we use export 'Entry' as symbol. This allows us to

; reference 'Entry' either in the linker .prm file

; or from C/C++ later on

XREF __SEG_END_SSTACK      ; symbol defined by the linker for the end of the stack

; variable/data section

Variables: Section

VAR_1:  dc.b $EC

Constants: Section

Port_s  equ $248

Ddr_s   equ $24A

Port_t  equ $240

MyCode: SECTION

main:

_Startup:

Entry:

LDAA    #$FF

STAA    Ddr_s     ; enabling all the leds for port s

LDAA    VAR_1     ; load VAR_1 into A

ANDA    #%10111111; clearing 6th bit of A

ORAA    #%00010000; set 4th bit of A

STAA    VAR_1     ; storing it back into VAR_1

Loop_0bit:  LDAB    Port_t    ; load value of Port t into B

ANDB    #%0000001 ; And B with %00000000

BEQ     Loop_0bit ; if equal 0 then branch back, so waits for bit bit to go high

Loop_0_low: LDAB    Port_t    ; load value of Port t into B

BITB    #%0000001 ; check if bit 0 goes low

BNE     Loop_0_low; if not equal 0 then branch

STAA    Port_s    ;

NOP

Lab 5.1.2:

Objective/Purpose:

The objective of this assembly code is to part is to implement Bit instructions.

Explanation:

The program first initializes DDR for Port s using bit set, then waits for switch 1 to go low, then high.

Results:

LEDs illuminate when the switch is turned from high to low then back to high, and they also illuminate when the switch is turned from low to high then back to low.

Code:

; Include derivative-specific definitions

INCLUDE 'derivative.inc'

; export symbols

XDEF Entry, _Startup, main

; we use export 'Entry' as symbol. This allows us to

; reference 'Entry' either in the linker .prm file

; or from C/C++ later on

XREF __SEG_END_SSTACK      ; symbol defined by the linker for the end of the stack

; variable/data section

Variables: Section

Constants: Section

Port_s  equ $248

Ddr_s   equ $24A

Port_t  equ $240

Mask    equ $FF

Mask1    equ $01

MyCode: SECTION

main:

_Startup:

Entry:

start:

BSET    Ddr_s, Mask  		; set all the leds for port s

loop_0high:  	BRCLR   Port_t, Mask1, loop_0high 	;branch if switch 1 is low

loop_0low:  	BRSET   Port_t, Mask1, loop_0low  	;branch if switch 1 is high

loop_0high2:  	BRCLR   Port_t, Mask1, loop_0high2 	;branch if switch 1 is low

BSET    Port_s, #%01000000

NOP

Lab 5.2:

Objective/Purpose:

The objective of this program is to add direction and speed controls to the stepper motor from Lab 3.

Explanation:

This program depends on which switch is high or low, the steeper motor's speed or direction varies.

Results:

When switch 1 is low, the motor rotates in a clockwise direction, and when switch 2 is low, the opposite is true. The motor should rotate slowly when switch 3 is low and quickly when switch 3 is high.

Code:

; Include derivative-specific definitions

INCLUDE 'derivative.inc'

; export symbols

XDEF Entry, _Startup, main, DelayCount

; we use export 'Entry' as symbol. This allows us to

; reference 'Entry' either in the linker .prm file

; or from C/C++ later on

XREF __SEG_END_SSTACK, Start      ; symbol defined by the linker for the end of the stack

Variables: Section

DelayCount:   ds.b    1

Constants: Section

Port_t  equ $240

port_p  equ $258

p_ddr:  equ $25A

array:  dc.b $0A,$12,$14,$0C,$0 		; sequence for clockwise

array2: dc.b $0C,$14,$12,$0A,$0		; sequence for counter-clockwise

Code:      Section

main:

_Startup:

Entry:

lds      #__SEG_END_SSTACK

LDAB     #%00011110

STAB     p_ddr

Check1:   BRCLR   Port_t, #$01, Check_2           ; branch to check_2 if bit 0 is 0

BRCLR   Port_t, #$02, Counter_clockwise ; branch to counter_clockwise if bit 1 is 0

BRA     Check1                          ; branch to check1 because both bits are 11

Check_2:  BRSET   Port_t, #$02, Clockwise         ; branch to clockwise if bit 1 is 1

BRA     Check1                          ; branch to check1 because both bits are 00

Clockwise:

Array_loop:LDX      #array

loop:     LDAA     1,x+				; load next value of X

CMPA     #0				; when end of the array is reached; reset it

BEQ      Array_loop			; reset the Array into X

STAA     port_p			; send the value of A to port_a

BRA      Check3

Counter_clockwise:

Array_loop2:LDX      #array2

loop2:    LDAA     1,x+			; load next value of X

CMPA     #0			; when end of the array is reached; reset it

BEQ      Array_loop2	; reset the Array into X

STAA     port_p           ; send the value of A to port_a

Check3:   BRSET   Port_t, #$04, speed_high         ; branch to high speed if bit 2 is 1

BRA     speed_low				   ; branch to low speed

speed_high:LDY   #15000					   ; delay for 15 milliseconds

STY    DelayCount

BRA    speed

speed_low:LDY   #60000					   ; delay for 60 milliseconds

STY    DelayCount

speed:    JSR       Start

Check1a:   BRCLR   Port_t, #$01, Check_2a           ; branch to check_2 if bit 0 is 0

BRCLR   Port_t, #$02, loop2 ; branch to counter_clockwise if bit 1 is 0

BRA     Check1a                          ; branch to check1 because both bits are 11

Check_2a:  BRSET   Port_t, #$02, loop         ; branch to clockwise if bit 1 is 1

BRA     Check1a                          ; branch to check1 because both bits are 00

;BRCLR   Port_t, #$01, loop

;BRA       loop2

nop

XDEF Start

XREF DelayCount

Start:    LDY     DelayCount

Delay:    DEY

BNE     Delay

RTS

Lab 5.3:

Objective/Purpose:

The program is designed to interface with an external system, likely a keypad and LEDs, and respond to user input. Specifically, it aims to read input from the keypad, process it, and display the results on a set of LEDs.

Explanation:

The code includes the following crucial sections: LEDs are initialized and set up for keypad input by 'Check_LED'; the keypad is read by 'Read', which runs the 'Sequence' subroutine; and then the keypad input is processed and the LEDs are controlled by 'LED1' and 'LED2'. 'Read_portU' obtains the value of 'Port_u' for 'Current' while 'Start_Delay' regulates time delays and 'Sequence' controls data processing. While 'Loop' and 'Match' are essential to lookup table processing, 'Key_pressed' and 'Not_pressed' manage button replies. 'END' denotes the conclusion of a subroutine. The precise functionality of this HC12 microcontroller code depends on certain hardware setups and connections.

Results:

The code is designed to interface with a keypad and LEDs. When a key is pressed, it processes the input and updates the LED display accordingly. The exact behavior and functionality would depend on the hardware setup and the specific requirements of the external system connected to the microcontroller. This code provides a framework for this interaction, but the specific behavior would be determined by the hardware and how the Port_u_array and Read_array are set up.

Code:

;**************************************************************

;* This stationery serves as the framework for a              *

;* user application. For a more comprehensive program that    *

;* demonstrates the more advanced functionality of this       *

;* processor, please see the demonstration applications       *

;* located in the examples subdirectory of the                *

;* Freescale CodeWarrior for the HC12 Program directory       *

;**************************************************************

; Include derivative-specific definitions

INCLUDE 'derivative.inc'

; export symbols

XDEF Entry, _Startup, main, Port_u, Port_u_array, Current, Count, Read_array

; we use export 'Entry' as symbol. This allows us to

; reference 'Entry' either in the linker .prm file

; or from C/C++ later on

XREF __SEG_END_SSTACK, Read_portU, Sequence     ; symbol defined by the linker for the end of the stack

; variable/data section

Variables: Section

Count:    	ds.b 	1

Current:  	ds.b 	1

Constants: Section

Port_u:  	equ $268

Ddr_u:   	equ $26A

Psr_u:  	equ $26D

Pder_u:  	equ $26C

Port_s:   	equ $248

Ddr_s:    	equ $24A

Port_u_array:	dc.b	$70, $B0, $D0, $E0,$0

Read_array:   	dc.b  $14, $88, $84, $82, $48, $44, $42, $28, $24, $22, $18, $14, $12, $81, $41, $21, $11,$0

MyCode: SECTION

main:

_Startup:

Entry:

CLI

LDS     #__SEG_END_SSTACK

BSET	  Ddr_u,	#$F0	; enabling all the bits 1-3 inputs and bits 4-7 outputs for port u

BSET	  Psr_u,	#$0F	; set bits 1-3 as pull up port u

BSET	  Pder_u,	#$0F	; activate pull up on pins 0-3

BSET	  Ddr_s,	#$FF	; activate all the Leds of Port_s

Check_LED:	LDAA 	  #$0			; Used to check if left or right LEDs should turn on

Read:

JSR	Sequence		; Reads the keypad

INCA               	; Increase A by 1 to indicate keypad was pressed

CMPA    #2			; checks if keypad was pressed twice

BEQ     LED2		; branch to display keypad pressed value on the left LEDs

LED1:       LDAB    Port_s		; Load the values of port_s into B

ANDB    #$F0		; Clearing right side LEDs

ORAB    Current		; Adding value of Keypad pressed to right side

STAB    Port_s		; Sending B to Port_s

BRA     Read     		; Loop back to read keypad again

LED2:       LDAB    Port_s		; Load the values of port_s into B

ANDB    #$0F		; Clearing left side LEDs

LSL     Current		; Shifting Current value to the 2nd Byte

LSL     Current

LSL     Current

LSL     Current

ORAB    Port_s		; Adding value of Keypad pressed to left side

STAB	  Port_s		; Sending B to Port_s

BRA     Check_LED	      ; Loop back to read keypad again and reset counter A to 0

NOP

XDEF Start_Dealy

Start_Dealy:	LDY 	#1000 	    ; Load delay counter of 30 milliseconds

Delay: 		    DEY 			        ; decrement Y

BNE   Delay 		  ; Branch to Delay if Y != 0

RTS

XDEF Sequence

XREF Port_u, Start_Dealy, Count, Read_array, Port_u_array, Current

Sequence:

LDX	  #Port_u_array	  	; load address of array into X

Next:         LDAA  0,X				; Load value of X into A

CMPA #$0 				; If end of the array is reached then reload the array back into X

BEQ Sequence

STAA	Port_u		    ; Load value of array into Port_u

INX				          ; Increment the register X

JSR	  Start_Dealy		; Start delay of 1 millisecond

BRA   Read_portU		; Branch to Read port_u

Read_portU:		LDAA	Port_u            ; Read value of Port_u into A

STAA  Current			; Save the value of Port_u for later use

ANDA	#$0F		        	; Ignore the 3 and 4 bytes

CMPA  #$0F				; Test if A = $0F / no key is pressed

BEQ 	Next            	; Zero then branch to next sequence

BRA   Key_pressed		; Branch to Key_pressed

Key_pressed:

LDAA  Port_u			; Read value of Port_u into A

ANDA  #$0F			; Ignore the 3 and 4 bytes

CMPA  #$0F			; Test if A = $0F / no key is pressed

BEQ   Not_pressed		; Zero then branch to key Not_pressed

BRA   Key_pressed		; Branch to back to Key_pressed

Not_pressed:  LDY		  #Read_array	; Load address of Lookup Table into Y

LDAA     Count   ; Load A with Count value

Loop:         LDAB    1, Y+   ; Load value from X+1 into B and X=X+1

CMPB #$0		; Check if Look up value is at the end of the array

BEQ END  		;

CMPB		Current ; Get Lookup Value

BEQ     Match

INCA            ; Increment A

BRA		  Loop		; Branch to loop

Match:

STAA    Current

END               RTS

Conclusion:

Boolean logic instructions, shift and rotate instructions, bit test and manipulation instructions, bit condition branch instructions, and switch debouncing were among the ideas I learnt. Debouncing inputs from mechanical switches was one of the principles that I struggled to comprehend. The lookup table on 5.3 was my main area of trouble.