Ravi Mann

ECE 362

Pre-Lab # 5

Introduction:

Purpose and Objective of the Lab:

The advanced I/O peripheral procedures were the emphasis of this lab. The goal of this lesson is to provide a firm foundation in logic, bitwise operations, and how to use them while working with complex I/O peripherals, including keypads. I will also discover how to employ these instructions for bit manipulation, if-else conditional execution, and looping.

Concepts Covered in the Lab:

- Implement Logic, Bit, and Shift Instructions

- Utilize Bit Test Instruction

- Master Debouncing Techniques

Lab 5.1.1: Logic Instruction

Objective/Purpose:

The objective of this lab is to familiarize students with logical instructions. Specifically, AND instruction to set bits by modifing the LED display's output based on the dipswitches' input. This requires using Var_1 in the code and manipulating it, as well as clearing bit 6 in accumulator A and restoring the value there.

Expected Results:

Var_1 value was read, modified, and stored as instructed by the program. Additionally, a waiting loop for switch 1 should be used, and when switch 2 goes low, the value of Var_1 should be shown on the LEDs. The rewritten software should also wait until switch 2 returns to a high state from a low state.

Code:

Variables:    	Section

VAR_1:         dc.b  		$EC

Constants:    	Section

Port_s        		equ   	$248

Ddr_s    	equ   	$24A

Port_t        		equ   	$240

MyCode:         SECTION

main:

_Startup:

Entry:

LDAA  	#$FF

STAA  	Ddr_s 		; enabling all the leds for port s

LDAA  	VAR_1

LDAB  	#$00

STAB  	Port_t 	; load Port_t with value of B

ANDA  	#$BF   	; AND BF = %10111111 and  EC = %11101100

STAA  	VAR_1  	; load %10101100 into VAR_1

ORAA  	#$10   	;OR operation of %00010000 and %10101100

STAA  	VAR_1   	; load %10111100 into VAR_1

BRCLR 	Port_t,#$02,start

BRA 	Loop_low

Loop_low:

BRSET 	Port_t,#$02,Loop_low 	;keep looping until bit 1 goes to low

Loop_high:

BRCLR 	Port_t,#$02,Loop_high 	;keep looping until bit 1 goes to high

BRA   	low_high 			;go to load values after going low and then high

low_high:

STAA  	Port_s    			; load value of A into Port_s

NOP

Lab 5.1.2: Bit Instructions

Objective/Purpose:

The objective of this assembly code is to create software that, depending on the number of times a certain switch has been flipped, waits for various delays in switches changing before turning on LED 6.

Expected Results:

The LED should turn on when the switch is flicked twice when the program begins with the switch in the high position. The switch must be turned three times if it is initially in a low position.

Lab 5.2: Steeper Motor

Objective/Purpose:

The objective of this program is to change the lab 3 stepper motor code to include functions for modifying the motor's rotational direction and speed. The software should contain a setting for moving or halting, rotating clockwise or counterclockwise, or the speed dependent on the dipswitch inputs in order to read the inputs from the switches. The speed needs to be adjustable as well.

Expected Results:

The input from the switches should allow for control of the stepper motor. Depending on whether switches are turned on or off, the dipswitches should be able to regulate the motor's speed, spinning direction, and ability to stop rotating.

Lab 5.3:

Objective/Purpose:

The software is created to respond to human input and interact with an external device, most often a keypad and LEDs. It seeks to read keypad input, process it, and then show the outcomes on a series of LEDs.

Expected Results:

The anticipated result of running this code on the suitable hardware configuration is the desired behavior outlined in the code comments. Processing keypad inputs, managing LEDs, and maybe interfacing with an external device are all included in this.