Ravi Mann

ECE 362

Post-Lab # 4

Introduction:

Introduction:

The purpose of this lab is to elevate your proficiency in assembly programming for the HCS12 microcontroller. By delving into more advanced concepts, we aim to equip you with the skills necessary to create intricate and well-structured programs.

Concepts Covered in the Lab:

- Implementing Control Structures:

  - This involves using constructs like loops (for, while) and conditional statements (if-else) to control program flow based on specific conditions.

- Basic Subroutines:

  - Create reusable blocks of code that can be called from different parts of a program. This promotes code modularity and efficiency.

- Stack Usage:

  - Understanding how the stack functions is essential for managing program execution and memory allocation. Also, gained insights into how to effectively use the stack.

- Software Delay Implementation:

  - Explored techniques to create delays in software, which can be crucial for synchronizing various parts of a program, especially in real-time applications.

- External Symbol Definition and Reference:

  - This concept involves compartmentalizing a project by separating different parts of the program into external symbols. This promotes modularity and simplifies project management.

Lab 4.1.1:

Objective/Purpose:

The objective of this lab is to enter, build, and execute specific programs. We will observe and record the value stored in accumulator A when the NOP instruction is reached. The programs will utilize for and while loops to traverse through an array.

Expected Results:

The program will initialize the data direction register of Port S for output. It will then iterate through the elements of the Array. If an element is non-zero, it will be output to Port S. Following the loop, there will be a delay of approximately 30 milliseconds. This entire process will continue indefinitely.

Explanation:

The program functions by first setting up Port S for output. It then enters a loop that iterates through the elements of the Array. For each element, it checks if it is non-zero. If so, it outputs the value to Port S. After completing the loop, a delay of about 30 milliseconds is introduced.

Results:

The code successfully configures Port S for output and cycles through the elements of the Array, outputting non-zero values. The program will run indefinitely, repeating this process in a continuous loop.

Observations:

The delay of approximately 30 milliseconds allows for a noticeable pause between iterations. The program effectively utilizes loops to iterate through the elements of the Array, providing a structured and controlled execution flow.

Code:

Constants:		Section

Port_s		EQU		$248		; Port s address

Ddr_s			EQU		$24A		; Port s ddr address

Array:		dc.b		$81,$42,$18,$24,$42,$0		; defining Sequence

Code:			Section

main:

_Startup:

Entry:

LDAB		#$FF 		    ; Load B with all bits of Port_s_ddr

STAB		Ddr_s		    ; Store value of B to ddr_s for LEDs

Array_loop:	  LDX		  	#Array	    ; Load address of Array into X

Loop:		  LDAA		1, X+		    ; Load value from X+1 into A and X=X+1

CMPA		#0		    ; Compare value of A with 0

BEQ 		Array_loop	    ; Branch to Array_loop if A = 0

STAA		Port_s	    ; Output value to Port_s

LDY		  	#30000	    ; Load delay counter of 30 milliseconds

Delay:	  DEY			      	    ; decrement Y

BNE	  		Delay		    ; Branch to Delay if Y != 0

BRA		  	Loop		    ; Branch to loop

Nop

Lab 4.1.2:

Objective/Purpose:

The objective of this assembly code is to control the output of Port S based on the state of Port T. The program will read the state of Port T and select either Array1 or Array2 for output to Port S. It will achieve this through conditional branching and loops. Additionally, a delay subroutine is used to introduce a controlled pause in the program execution.

Explanation:

Data direction registers for Ports S and T are initialized. Reads Port T's current status and saves it in Val_T. Chooses either Array1 or Array2 for output to Port S based on the state of Port T. performs a check on each element in a loop across the chosen array. Outputs it to Port S if it is non-zero. brings in a delay of about 30 milliseconds. carries on endlessly in this manner.

Results:

The second part of the code likely follows a similar structure but with a different set of arrays and conditions based on Port T's state. This program will continuously run, selecting and outputting sequences based on Port T's state. The delay ensures controlled execution.

Code:

Variables:		Section

Array1:		dc.b	$81,$42,$18,$24,$42,$0				; defining Sequence 1

Array2:		dc.b	$80,$40,$20,$10,$08,$04,$02,$01,$0		; defining Sequence 2

Val_T:		ds.b	2

Constants:		Section

Port_s		EQU		$248		; Port s address

Ddr_s			EQU		$24A		; Port s ddr address

Port_t		EQU		$240		; Port t address

; code section

MyCode:     SECTION

main:

_Startup:

Entry:

LDAB	  	#$FF 		; Load B with all bits of Port_s_ddr

STAB	  	Ddr_s		; Store value of B to ddr_s for LEDs

LDAA	  	Port_t	; Load A with value of Port_t

CMPA	  	#0		; Compare value of A with 0

BEQ		A1_loop	; Branch to A1_loop if A = 0

BRA		A2_loop	; Branch to A1_loop

A1_loop:	LDX		#Array1	; Load address of Array1 into X

Loop1:	LDAA	   	1, X+		; Load value from X+1 into A and X=X+1

CMPA		#0	  	; Compare value of A with 0

BEQ 		Entry		; Branch to Entry if A = 0

STAA		Port_s	; Output value to Port_s

BRA		Start_delay

A2_loop:	LDX		 #Array2		; Load address of Array1 into X

Loop2:	LDAA		 1, X+		; Load value from X+1 into A and X=X+1

CMPA		 #0	  		; Compare value of A with 0

BEQ 		 Entry		; Branch to Entry if A = 0

STAA		 Port_s		; Output value to Port_s

Start_delay:LDY		  #500000		; Load delay counter of 30 milliseconds

Delay:	DEY				      ; decrement Y

BNE		  Delay		; Branch to Delay if Y != 0

LDAB		  Port_t		; Load value of Val_T into B

CMPB	  	  #0	  		; Compare Value of B with 0

BEQ	    	  Loop1		; Branch to Loop1 if B = 0

BRA		  Loop2		; Branch to Loop2

END

Nop

Lab 4.2:

Objective/Purpose:

The objective of this program is to create a lookup table-based system for controlling LEDs based on the state of switches. It aims to read the switch input continuously, use a lookup table to map each switch state to a specific LED output value, and display the corresponding LED pattern on the LEDs connected to the microcontroller.

Explanation:

This program continuously monitors the state of switches connected to Port T. It uses a lookup table to determine the corresponding LED pattern and displays it on Port S. The program runs in an infinite loop, providing real-time interaction between switches and LEDs.

Results:

The program will initialize Port S for output. It will then continuously monitor the state of the switches connected to Port T. Whenever the switch state changes, the program will use the lookup table to determine the corresponding LED pattern and display it on Port S. The program will run in an infinite loop, providing real-time interaction between switches and LEDs.

Code:

XDEF	Entry

XREF	__SEG_END_SSTACK

Variables:		Section

Lookup:		dc.b	$12,$3A,$61,$86,$18,$43,$6F,$C3,$24,$4B,$7A,$D6,$36,$51,$92,$F1

Constants:		Section

Port_s		EQU		$248		; Port s address

Ddr_s			EQU		$24A		; Port s ddr address

Port_t		EQU		$240		; Port t address

; code section

MyCode:     SECTION

main:

_Startup:

Entry:

LDAB		#$FF 		; Load B with all bits of Port_s_ddr

STAB		Ddr_s		; Store value of B to ddr_s for LEDs

Loop:		LDX		#Lookup	; Load address of Lookup Table into X

LDAB		Port_t	; Load B with Value of Port_t

ABX				; Add A to X to find address of

LDAA		0, X		; Get Lookup Value

STAA		Port_s	; Send value of A to Port_s

BRA		Loop		; Branch to loop

END

Nop

Lab 4.3:

Objective/Purpose:

The objective of this lab is to develop an assembly program capable of searching for a specific value, represented by $dd, within a predefined lookup table. The program is designed to determine the index of this value within the table. If a match is found, the program should return the index value in the accumulator (A). Conversely, if no match is found, it should load the value $FF into the accumulator.

Explanation:

The program employs a straightforward linear search as its methodology. It progressively checks each element of the lookup table until it finds a match or reaches the end of the table. The loop iterates again, increasing the index (B) each time. This index is used to keep track of a place within the Lookup table. The software effectively controls the search process and provides detailed information on whether a match was found. The software is structured with the appropriate labels and comments, which makes it easier for developers to read and understand. With the use of conditional branches (BEQ), which are crucial for controlling program flow, the program may make choices based on the results of the comparisons.

Results:

The program will locate the index of the value stored in Val and return it to the accumulator (A) if the value is located in the Lookup table. If the lookup table does not include the value contained in Val, the program will search the whole table in vain. The accumulator (A) will be loaded with $FF in this situation.

Code:

XDEF	Entry

XREF	__SEG_END_SSTACK

Variables:		Section

Val:      ds.b  1

Constants:		Section

Count     dc.b  	0

Lookup:   dc.b	$eb,$77,$7b,$7d,$b7,$bb,$bd,$d7,$db,$dd,$e7,$ed,$7e,$be,$de,$ee, $0

; code section

MyCode:     SECTION

main:

_Startup:

Entry:

LDX	  #Lookup			; Load address of Lookup Table into X

LDAB    Count   			; Load B with Count value

Loop:       LDAA    1, X+   			; Load value from X+1 into A and X=X+1

CMPA		Val   		; Get Lookup Value

BEQ     Match

CMPA    #0

BEQ     No_Match

INCB            		 	; Increment Y

BRA	  Loop			; Branch to loop

Match:      TFR     B,A

BRA     Done

No_Match:   LDAA    $FF

Done:

Nop

Lab 4.4:

Objective/Purpose:

The objective of this code structure is to create a modular program for controlling a stepper motor. By isolating the delay loop in a separate file, the code becomes more organized. Additionally, the use of the variable DelayCount provides adaptability, allowing for the adjustment of delay duration, making the program suitable for various applications.

Explanation:

External and reference symbols are defined for linking using the XDEF and XREF directives. DelayCount: ds.b 1: DelayCount is a variable that indicates how many times a delay has occurred, enabling the length to be changed.

Certain bits in the B register are set by LDAB #%00011110, perhaps enabling port P for output. The function Array_loop creates a loop that iterates across the array. The loop loads a value from the array into A, checks it against 0, and returns to the Array_loop if the value is not zero. If the value is not zero, port_p is updated, and the procedure Start is invoked.

A subroutine called Start starts a delay loop. DelayCount establishes how long the delay lasts.

Result:

This program controls a stepper motor using a delay loop and a lookup table (array). The program will iterate through the table, outputting non-zero values to port_p, and then initiating a delay determined by DelayCount.

Code:

; Include derivative-specific definitions

INCLUDE 'derivative.inc'

; export symbols

XDEF Entry, _Startup, main, DelayCount

; we use export 'Entry' as symbol. This allows us to

; reference 'Entry' either in the linker .prm file

; or from C/C++ later on

XREF __SEG_END_SSTACK, Start      ; symbol defined by the linker for the end of the stack

Variables: Section

DelayCount:   ds.b    1

Constants: Section

port_p equ $258

p_ddr: equ $25A

array: dc.b $0A,$12,$14,$0C,$0

Code:      Section

main:

_Startup:

Entry:

lds      #__SEG_END_SSTACK

LDAB     #%00011110

STAB     p_ddr

Array_loop:LDX      #array

loop:     LDAA     1,x+

CMPA     #0

BEQ      Array_loop

STAA     port_p

JSR       Start

BRA       loop

nop

XDEF Start

XREF DelayCount

Start:    LDY     DelayCount

Delay:    DEY

BNE     Delay

RTS

Conclusion:

Concepts Learned:

Implementing regulate Structures: I learnt how to effectively regulate program flow depending on specified situations using loops (for, while) and conditional expressions (if-else). Having this information will be crucial for developing programs that are more intricate and well-organized.

Basic Subroutines: I was successful in creating callable code reusable blocks that may be used throughout a program. This encourages code efficiency and modularity, two key components of effective programming.

Stack usage: Controlling program execution and memory allocation requires an understanding of how the stack works. I now understand how to use the stack to organize programs more efficiently.

Implementing software delays: I looked at many methods for doing this. This information will be very important for synchronizing various software components, especially in real-time applications.

Definition and Use of External Symbols I discovered the value of compartmentalizing a project by breaking the program's components down into external symbols. Project management is made easier, and modularity is encouraged.

These ideas collectively offer a solid framework for future development of programs that are more intricate. Now that I know how to use stacks and external symbols, I can create programs with sophisticated control structures, reusable subroutines, and better organization.

Concepts Not Fully Understood:

All the topics addressed were clearly described, and I was able to understand them all. External and reference symbols are established for linking using the XDEF and XREF directives, which was one idea that was challenging to understand.

Difficulties Encountered:

It was not difficult to learn about external and reference symbols. Understanding was made easier by the detailed explanations and examples given. Project management was made simpler after it was understood how to separate a project by breaking apart distinct elements of the program into external symbols. This idea turned out to be a useful tool for building more structured, modular programs.

Overall, this lab gave me practical experience in assembly programming that was helpful, and I now have greater faith in my abilities to use the HC(S)12 microcontroller in future projects.