Ravi Mann

ECE 362

Pre-Lab # 4

Introduction:

Purpose and Objective of the Lab:

The lab aims to build on the basic assembly programming concepts previously introduced, providing students with a more comprehensive understanding of how to write more complex and structured programs in HCS12 assembly language.

Concepts Covered in the Lab:

- Implementing Control Structures

- Basic Subroutines

- Stack Usage

- Software Delay Implementation

- External Symbol Definition and Reference

Lab 4.1.1:

Objective/Purpose:

Enter, build, and run specific programs. The value in accumulator A when the NOP instruction is reached will be observed and recorded. The program will utilize for/while loops to traverse through the array.

Expected Results:

The program will likely configure the data direction register of Port S to enable it for output. It will then loop through the elements of the Array. For each element: If the element is not zero, it will output the value to Port S. After the loop, there's a delay of approximately 30 milliseconds. The program will repeat this process for an indefinite period.

Code:

XDEF	Entry

XREF	__SEG_END_SSTACK

Variables:		Section

Array:		dc.b	$81,$42,$18,$24,$42,$0		; defining Sequence

Constants:		Section

Port_s		EQU		$248		; Port s address

Ddr_s			EQU		$24A		; Port s ddr address

Code:			Section

Entry:

LDAB		#$FF 		; Load B with all bits of Port_s_ddr

STAB		Ddr_s		; Store value of B to ddr_s for LEDs

Array_loop:		LDAX		#Array	; Load address of Array into X

Loop:			LDAA		1, X+		; Load value from X+1 into A and X=X+1

CMPA		#0		; Compare value of A with 0

BRA 		Array_loop	; Branch to Array_loop if A = 0

STAA		Port_s	; Output value to Port_s

LDY		#30000	; Load delay counter of 30 milliseconds

Delay:		DEY				; decrement Y

BNE		Delay		; Branch to Delay if Y != 0

BRA		Loop		; Branch to loop

END

Nop

Lab 4.1.2:

Objective/Purpose:

The objective of this assembly code is to control the output of Port S based on the state of Port T. Read the state of Port T. Depending on the state of Port T, select either Seq1 or Seq2 to be sent to Port S. Send out the selected sequence to Port S in a controlled manner using a loop and a delay subroutine.

Expected Results:

The program will initialize the data direction registers for Ports S and T, allowing them to be used for input and output. It will then enter the main loop where it checks the state of Port T. If Port T is zero, Seq1 will be selected; otherwise, Seq2 will be selected. The selected sequence will be sent to Port S using a loop. Each element of the sequence will be output to Port S, one at a time, with a delay in between.

Code:

XDEF	Entry

XREF	__SEG_END_SSTACK

Variables:		Section

Array1:		dc.b	$81,$42,$18,$24,$42,$0				; defining Sequence 1

Array2:		dc.b	$80,$40,$20,$10,$08,$04,$02,$01,$0		; defining Sequence 2

Val_T:		ds.b	2

Constants:		Section

Port_s		EQU		$248		; Port s address

Ddr_s			EQU		$24A		; Port s ddr address

Port_t		EQU		$240		; Port t address

My_Code:			Section

Entry:

LDAB		#$FF 		; Load B with all bits of Port_s_ddr

STAB		Ddr_s		; Store value of B to ddr_s for LEDs

LDAA		Port_t	; Load A with value of Port_t

STDA		VAL_T		: Store value of A into VAL_T

CMPA		#0		; Compare value of A with 0

BEQ		A1_loop	; Branch to A1_loop if A = 0

BRA		A2_loop	; Branch to A1_loop

A1_loop:		LDAX		#Array1	; Load address of Array1 into X

Loop1:		LDAA		1, X+		; Load value from X+1 into A and X=X+1

CMPA		#0		; Compare value of A with 0

BRA 		Entry		; Branch to Entry if A = 0

STAA		Port_s	; Output value to Port_s

BRA		Start_delay

A2_loop:		LDAX		#Array2	; Load address of Array1 into X

Loop2:		LDAA		1, X+		; Load value from X+1 into A and X=X+1

CMPA		#0		; Compare value of A with 0

BRA 		Entry		; Branch to Entry if A = 0

STAA		Port_s	; Output value to Port_s

Start_delay:	LDY		#30000	; Load delay counter of 30 milliseconds

Delay:		DEY				; decrement Y

BNE		Delay		; Branch to Delay if Y != 0

LDAB		Val_T		; Load value of Val_T into B

CMPB		#0		; Compare Value of B with 0

BEQ		Loop1		; Branch to Loop1 if B = 0

BRA		Loop2		; Branch to Loop2

END

Nop

Lab 4.2:

Objective/Purpose:

The objective of this program is to create a simple lookup table-based system for controlling LEDs based on the state of switches. The purpose is to read the switch input continuously, use a lookup table to map each switch state to a specific LED output value, and display the corresponding LED pattern on the LEDs connected to the microcontroller.

Expected Results:

The Lookup Table will be unitized to connect to the LEDs of Port S. Whenever you change the state of the switches, the program should immediately update the LED pattern to reflect the new switch state. The program will run in an infinite loop, continually monitoring the switches, and updating the LEDs as necessary.

Code:

XDEF	Entry

XREF	__SEG_END_SSTACK

Variables:		Section

Lookup:		dc.b	$12,$3A,$61,$86,$18,$43,$6F,$C3,$24,$4B,$7A,$D6,$36,$51,$92,$F1

Constants:		Section

Port_s		EQU		$248		; Port s address

Ddr_s			EQU		$24A		; Port s ddr address

Port_t		EQU		$240		; Port t address

My_Code:			Section

Entry:

LDAB		#$FF 		; Load B with all bits of Port_s_ddr

STAB		Ddr_s		; Store value of B to ddr_s for LEDs

Loop:			LDAX		#Lookup	; Load address of Lookup Table into X

LDAB		Port_t	; Load B with Value of Port_t

ABX				; Add A to X to find address of

IDAA		0, X		; Get Lookup Value

STAA		Port_s	; Send value of A to Port_s

BRA		Loop		; Branch to loop

END

Nop

Lab 4.3:

Objective/Purpose:

The objective of this program is to search for a specific value ($dd) in a lookup table. The program aims to determine the index of the value in the table. If a match is found, it should return the index value in the accumulator (A). If no match is found, it should load $FF into A.

Expected Results:

If $dd is present in the lookup table, the program will identify its index. If $dd is not present in the table, the program will reach the end of the table without finding a match.

Lab 4.4:

Objective/Purpose:

The objective of this code structure is to create a nodular program for controlling a stepper motor. By placing the delay loop in a separate file, I can better organize my code. Using the variable for DelayCount  allows me to adjust the delay length, making the program adaptable for different applications.

Expected Results:

The delay loop in a separate file is used to introduce a delay in the program execution. The number of iterations in the delay loop is determined by DealyCount. By adjusting the value, I can change the duration of the delay, affecting the behavior of the stepper motor.